Project Summery and Scope of Work

Cherokee Nation Supreme Court Museum

08/03/2023

The building is constructed of two-story multi-wythe brick on a stone block foundation with a wood deck floor over wood joists. The roof is a wood framed hip roof with wood shingles. There is a small concrete porch at the front entrance of the building and a concrete ramp at the back entrance. The building is not protected by an automatic sprinkler system.

Work on the project will include minor water damage repairs to interior finishes, masonry repairs, and painting. The existing wood windows will all be replaced with aluminum clad wood windows, and the drainage and site grading will be reworked to allow for the water to drain away from the site properly. The roof will have the shingles replaced.

There will be an extensive new drain system installed to ensure water is taken away from the building. New gutters and downspouts will be added and connected to the new drain system, and the adjacent building will have its new gutters and downspouts tied into the drain system. The drain system will also include a catchment basin, trench drain, and new perforated drain. The new drain systems will daylight through the sidewalk on the east end of the site and will require minor concrete work.

Alternate #1 – Replace existing windows with new all wood windows in lieu of aluminum clad.