### CHEROKEE NATION SPEAKERS SERVICES

O FFICE: 918 .207 .4904

## Home Rehabilitation/Replacement Site Directions

Name: _ Donald Lee Sevenstar 	_ Telephone: none 	_

Address: _ 474106 E 1007 RD 	_

_Muldrow, OK 74948 	_

County: _Sequoyah 	_ Year Built:	60's---

GPS Coordinates: 		35.4804575,-94.665254 	_

- Drive Directions:	From Muldrow OK

### Head North on OK-64B 10.5 miles

- Turn right onto OK-101 East 2.1 miles

- Turn right onto E 1007 RD

- Destination on Right 350 ft small white house

Cost Estimator/ Inspector _Lonnie Ragland

_	Date:

7/24/25

WorkWrite-Up and Bid Document/By Area

### Donald Lee Sevenstar

#### 474106 E 1007 Rd Muldrow, OK	7-4948

Contractor Name:

Contractor Signature:

(000) 000-0000	Tllil' document mw,t he .\ignetl to he" Vlllid bid.

PROJECT TYPE:	Rehabilitation	Bid Due Date:

Area	Work Description I Comments	Gen. Spec.	Qty Unit	Item Bid

| EXTERIOR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 Soffit/Repair. Repair damaged soffit/See comments. | Div. # 6 . P | SOSF I I |  |  |  |
| replace any and all rotted or damaged soffits and paint to match 2 coats |  |  |  |  |  |
| INTERIOR |  |  |  |  |  |
| 2 | Div. # 9.D |  |  |  |  |

Drywall/Ceiling/Install new 5/8" drywall on ceiling/Finish completely. Work shall include tape, bed and texture to match existing surfaces.

150SF	I	I

ROOF

repair and or replace leak damaged drywall a-nd-pa-int -2 c-oa-ts  -------------------

3	Div. # 6 .P

Fascia/Replace/1" x 4". Replace Fascia/1" x 4".

replace rotted fascia paint to match

50LF	I	I

Print Date:  Thursday, July 24, 2025	474106 E 1007 Rd Muldrow, OK	7•4948	Page 1 of 3

Area	Work Description  I :c  ommentsI	Gen. Spec.	Qty Unit	Item Bid

ROOF

4	Div. # 7.F.	220LF

Flashing/Drip Edge/Replace. Replace drip edge flashing. Match existing as closely as possible.

| 5 | Div. # 7 . F |  |
| --- | --- | --- |
| Flashing/Replace. Replace flashing. replace needed flashing |  |  |
| 6 | Div. # 7 .A. |  |

approx 220 ft drip edge replacement		-

I		I

Roofing/Architectural/30 Year Warranty. Install fiberglass shingles/Minimum 30 year warranty/Use Owens Corning "Classic" or written approved equal. Remove all shingles, felt, pipe jacks and flashing.

install shingles, starter appro x. 80If . install ndge approx 951f ,  install felt 20sq, install ice and

water shield, replace required decking haul off and dispose of debris		-

40LF	 I	I

20Sq	 I	I

I                  I

Div. # 7 .C	20Sq

Roofing/Strip/Built-Up. Strip roof/built-up.

I I

tear off old roofing and rotted sheeting			-

- Div. ff 7 .A	200SF

Sheathing/3/4" COX/Replace. Replace existing sheathing/Use 3/4" CDX. replace up to 200 sf of sheeting

Print Date:  Thursday, July 14, 1025	.f7'106  E 1007 Rd Muldrow, OK	7-.f9.f8	Page2of3

"P; r

# •

•	• •

,

'

""