Cherokee Nation Cultural and Economic Development along with Cheroke Nation Businesses is seeking General Contractors to perform waterproofing and installation of a French drain system along a basement wall, tuck pointing of brick veneer, repair and stabilization of rock retaining wall, grading and seed, TPO roof removal and replacement and interior wall repair and finish at in Claremore, Oklahoma.  The project is lump sum and is to include all labor and material necessary to complete the work.

Scope of Work:

- Remove large tree on the East side of the house prior to excavation

- Excavate along east side of house to bottom of footing approximately 85 feet

- Remove and clean all debris on wall down to footing

- Install GC-250 and wallboard

- Install 6-inch drainpipe with filter fabric along base of wall and daylight towards the road

- Backfill with 1-inch clean rock maintains rock thickness of 2 to 3 feet from wall to surface

- Demo, salvage and reinstall approximately 31 feet of rock wall that is leaning and

- Stabilize footing

- Install 8-inch CMU block to help support the wall behind the rock

- Reinstall rock wall to set in front of CMU

- Finale grade and seed of disturbed area.

- Tuck point existing brick veneer as identified by owner

- Install a brick veneer sealer on all exterior walls of the house and garage

- Remove and replace with new TPO roofing membrane. Identify area of additional damage to decking.  This will be removed and replaced as needed.

- Install additional flashing as required

- Repair interior walls, windows and ceilings as identified by owner

- Contractor is responsible for all permits, safety barriers debris removal and cleaning up

- Site is to be cleaned of trash and debris