PACE Ornamental Fence Specifications

Manufactured in the USA

Equal to or better than Ameristar Montage Plus, (Classic style three rail steel ornamental fence, Black in color).

Follow all the Manufacturers’ installation instructions.

Fencing shall have a minimum of a 20-year manufacture  warranty.

Pickets: .75"sq x 18ga

Rails: 1.4375" x 1.5" x 14ga

Posts: 2.5"sq x 16ga

Heights: 4'

IBC Compliant Configuration:

- 3 rails

- 2.5"sq. x 14 ga. min posts

- All brackets (flat mount and swivel must be thru-bolted)

- For plated posts: 8" x 8" min plates with gussets

3 Rail Panels

- 4' height

- Standard bottom rail

- 4" standard picket airspace

Pricing shall include all materials and installation costs for a 100% complete project.