Project Scope

This SOW covers the following activities, deliverables and or requirements.

CNE Provisions:

- Content computer with HDMI output

Tasks:

- Labor – Contractor will complete the following items.

  - Removal and disposal of existing two sided RGB LED message board and associated interface and control components.

    - Existing sign is 20mm pitch and 112x196 resolution.

  - Installation of new outdoor LED message board and associated interface and control components.

    - LED Sign:

- Daktronics or Watchfire

- Outdoor rated, fully sealed and weather proof

- RGB, 16mm Pitch or smaller

- Front serviceable

- 5-year parts and labor warranty

- Horizontal viewing angle must be greater than or equal to 140 degrees

- Vertical viewing angle must be greater than or equal to 70 degrees

- Panels must be self-venting and be able to maintain proper operating temperatures without the need of air conditioning

- Lifetime of panels must be 50,000 hours or greater at full brightness

- 10% replacement panels for future support.

- Photocell and timer on off capability

    - Sign Interface: Used to display custom content on the sign

- Must be able to receive an HDMI input

- Rack mountable

- Must be able to be located away from the sign in the casino Data Center or IDF, and connect to the sign via CNE provided fiber.

- Conditions

  - Work schedule will be coordinated with CNE.

  - Contractor is responsible for keeping all work areas clean.

- Testing

  - Final testing of sign will be done with CNE marketing content and in presence of CNE IT and property management personnel prior to final sign off of completion acceptance.

Deliverables:

- New, double sided, 16mm pitch, RGB LED message board mounted to pylon sign.

- Warranty Letter to include equipment and installation warranty dates, if applicable.

- As built drawings

- Equipment cut sheets on all components

Key Requirements: