SCOPE OF WORK

The kitchen floor to be renovated is located within the Hard Rock Tulsa Casino & Hotel on the first floor. The kitchen flooring to be renovated is in the space formerly known as “Toby Keith’s I Love This Bar & Grill”. The kitchen floor is presently covered with quarry tile with 6” quarry tile base. Contractor is responsible for its own quantity estimates. Floor plan is provided as a courtesy only. Contractor shall provide a turn-key flooring solution based on the following minimum requirements:

- Demo all existing quarry tile and associated base. Legally dispose of all waste materials.

- Grind all mortar from slab on grade and prep floor for an epoxy flooring.

- Install seamless epoxy flooring system and seamless, integrated epoxy, base (6” high).

  - Approved epoxy flooring products.

    - Poly-Crete MDB Dur-A-Flex

    - Approved equal. Substitutions must be submitted and approved seven (7) days prior to bid due date.

- Install seamless epoxy flooring system in strict compliance with manufacturer’s installation instructions and guidelines.

- It is the Contractor’s responsibility to ensure its employees are cleared to work onsite by checking in at the security desk daily and each employee receiving a vendor pass for that day.

- Contractor shall maintain a safe and clean worksite that is acceptable to the Owner.

- Contractor will advise of any loud disruptions that may impact Owner’s business and shall receive Owner approval prior to operating any disruptive equipment. Generally speaking, disruptive equipment will be allowed Monday thru Wednesday, 0700 – 1700 hours and Thursdays, 0700 – 1200 hours. Disruptive work outside of those hours will require Owner approval.

- Contractor shall provide a one (1) year labor and material warranty in addition to the manufacturer’s base warranty.

- This project is materials tax exempt.