Scope of Work – Fire Suppression Room Addition – Sequoyah’s Cabin

Cherokee Nation Culture and Tourism is seeking qualified general contractors to construct a 10' x 10' building adjacent to the rock structure housing Sequoyah’s original cabin in Sallisaw Oklahoma.

The work will consist of the following:

- clear, grub, over excavate and fill

- perimeter foundations and 5” slab

- heavy gauge metal stud frame, insulation at walls/ceiling,  wood truss and deck

- sheetrock ceiling at 8’

- exterior sheathing with weather barrier

- exterior rock siding match existing structure, rough lumber fascia to match existing structure

- shingled roof to match existing building

- Interior sheetrock, taped and mudded.

- rubber base, exposed sealed concrete

- 3 x 7 exterior grade HM Door, frame and weather seal hardware

- paint door/frame

- 1 ton mini split heat/cool electric unit

- electrical from existing panel, receptacles and lights

- 1 electrical dedicated circuit for the fire suppression equipment

- included A&E for drawing and engineering

Option for exterior siding to consist of rough sawn weathered barn wood in lieu of rock

The contractor is responsible for all permits and required approvals.  This is a lump sum project to include all associated costs.