Cherokee Nation Property Management is seeking qualified excavation contractors to bore 2 sections of asphalt driveways and trench/cover/cleanup 1 section of electrical ditch.

Road bore Scope of work is as follows;

- Bore one 120ft span under asphalt

- Pull two 2” Poly in bore hole

- Bore one 80ft span under asphalt

- Pull two 2” Poly in bore hole

- Trench approximately 150ft of electrical ditch

- Allow CPM to install piping and wire in ditch

- Unearth piping stubbed out from electrical poles to allow conduit connection

- Cover and cleanup all holes/trenches

Contractor will be expected to follow all state and federal laws related to a safe working environment. Work areas must be cordoned off.

Bore Specifications;

- 120ft bore will be one 2”Poly and one 2” Poly sleeve(future)

- 80ft bore will be one 2”Poly and one 2” Poly sleeve (future)

- CPM will provide all PVC piping and electrical supplies/perform  electrical install

- Contractor to supply 2”Poly for Bores

Any scheduling and coordination should be made with Property Management MEP department.